AEL.MF.07 B 5(90) AMPERE ACTIVE SINGLE-PHASE COUNTERS

TECHNICAL SPECIFICATIONS

1) Model Name: 			AEL.MF.07_B

2) Definition of Counter : 		Outdoor Single-Phase Active Counter with 1 Phase, 2 Wires, 10 Tariffs,

Demand meter

3) Accuracy Class:		Mode B ( Class 1 )

4) Reference Voltage: 		230V

5) Frequency:	 		50Hz

6) Take-off Current: 		20mA

7) Minimum Current : 		0.25A

8) Reference Current: 		5A

9) Maximum Current : 		90Ampere ( MS58 Brass Alloy )

10) IP Class: 			IP54

11) Protection Class: 		2

12) Relative Humidity Rate :	 %95

13) Operating Temperature: 	-40 , +80 °C

14) Mechanical Environment: M1 	(Acc. to 2004 / 22 / EC directive )

15) Electromagnetic: 		 E2 (Acc. to 2004 / 22 / EC directive)

16) Voltage Circuit: 		< 1.2W (in standards  2W )

17) Current Circuit: 		< 0.1W (in standards  1W )

18) ESD: 			8KV with contacts, 15KV by air

19) Impulse Voltage: 		6kV

20) Counter Constant: 		1600imp/kWh

21) LCD:			 6+3

22) Communication Rate: 		300 – 4800 ( IEC 62056-21 Mode C )

23) Interface I:  			Optical Port

25) System Battery Life: 		10 years

26) RTC Battery Life: 		10 years

27) Energy Records: 		12-Month Index Information, 12-Month Demand Information,

28) Intervention: 	12-Month Electric Terminal Cover Information, Housing Opening Date Information

29) Wide Load Profile:  		Voltage load profile record for 45days in case of 15 minutes, 180 days in 						case of 60 minutes.

30) Wide Event Records:		Energy Interruptions Event Records

Date Hour Day Change Event Records

Tariff change time event record

Electric Terminal Cover Opened, Closed

Password Change Records

Hour Error Event

Microprocessor Hard Reset Time

31) Battery Records: 		Customer mode switching date, Second Information of Counter with

Energy, Second Information of Counter without Energy, Battery Percentage, Temperature Information

32) ID Information: 		Model, Serial No, Manufacturing Date, Calibration Date, Demand Measurement Duration, First Tariff Loading Date,

33) Security Records: 		Upper Cover, Terminal Cover, Memory Error, System Battery Error, 							RTC Battery Error etc.

34) Relevant Standards: 		MID Certified ( EN 62052-11 , EN 62053-22 , EN 62053-23 ),

CE Certified ( 2004 / 22 / EC )

35) Regulation: 			Conformity with Tedaş Specifications consisting of 38 Articles.