AEL.MF.15  10(80)AMPERE SINGLE-PHASE COUNTERS WITH PREPAID

(PAYMETERS)

TECHNICAL SPECIFICATIONS

1) Model Name: 		AEL.MF.05

2) Definition of Counter : 	Outdoor, Prepaid Counter with 1 Phase,  2 Wires,

4Tariffs, Demand meter

3) Accuracy Class:		Class 1

4) Reference Voltage: 		230V/400 V

5) Wide Input Voltage : 	130V ( Minimum ) – 280V( Maximum )

6) Frequency: 		 	50Hz

7) Take-off Current: 		20mA

8) Minimum Current : 		250 mA

9) Reference Current: 		5A

10) Maximum Current : 	80 A

11) IP Class: 			IP54

12) Protection Class: 		2

13) Relative Humidity Rate :	 %95

14) Operating Temperature: 	-40 , +80 °C

15) Mechanical Environment: 	M1 ( Acc. to 2004 / 22 / EC directive )

16) Electromagnetic: 		E2 (Acc. to 2004 / 22 / EC directive)

17) Voltage Circuit: 		< 1W (should be < 2W  in standards)

18) Current Circuit: 		< 0.2W (should be < 1W in standards)

19) Total power: 		<1.2W (total power consumption per phase)

20) ESD: 			8KV with contacts, 15KV by air

21) Impulse Voltage: 		6kV

22) Counter Constant: 		1600imp/kWh

23) LCD:			 5+3

24) Communication Rate: 	300-4800 ( IEC 62056-21 Mode C )

25) Interface A: 		Optical Port (300-4800) (IEC 62056-21 Mode C)

26) Interface B:			RS485 ( 300 – 9600 ) ( IEC 62056-21 Mode C )

27) System Battery Life:	10 years

28) Cutting structure: 		Inner Opening Cutting unit

29) Security Records: 		Upper Cover, Electric Terminal Cover

30) Data logging: 		1 YEAR

31) Relevant Standards:		TS EN 62056-11 , TS EN 62053-21

32) Relevant Documents: 	MID

33) Technology:   	Contactless communication is provided through the RFID technology (Optional)

35) Alarm Circuit:		Cancellable acoustics alarm circuit (Optional)

36) Contour back-up: 		Available (Optional)

37) Compatibility: 	Since it is possible to make writing at required sectors, more than one application can be activated by the same card at the same moment.   (Optional)

38)  Credit: 	It is made sure that the meter does not make cutting at the required periods of time through the contour credit opportunity. (Optional)

39) Easy to use:  	Easy installation and easy to use feature by means of an Accounting Program. (Optional)